# OS Installation and Initial Configurations for Raspberry Pi

Created by: Lochana Marasinghe

Stereo pi pinout: https://stereopi.com/blog/synchronizing-photos-two-stereopi-boards

- Install Raspberry Pi Bullseye (Raspbian GNU/Linux 11 (bullseye)) with armv7l architecture

- The following configurations were initially set.

For each Raspberry Pi, this was continued.

- Connect the network cable from your PC to the Raspberry Pi and connect it via PuTTY

Connect your PC to Wi-Fi and enable the sharing of the network for other connected devices,

Python version: Python 3.9.2

- Run

sudo raspi-config

- Enable camera,i2c,SSH,OpenGL,SPI

- Change the timezone to America>vancouver

- Run sudo reboot

- Connect again via SSH

- Check the date using date command

- Start VNC server

vncserver-virtual

- For the pi cameras put the dt-blob.bin file to the /BOOT folder.

sudo mv  /home/pi/Desktop/dt-blob.bin /boot

- Make sure you have an internet connection in raspberry pi. You can use the command below to check.

curl https://linuxconfig.org

- Install open cv

sudo apt-get -y install ca-certificates

sudo apt-get update

sudo apt-get install build-essential cmake pkg-config libjpeg-dev libtiff5-dev libjasper-dev libpng-dev libavcodec-dev libavformat-dev libswscale-dev libv4l-dev libxvidcore-dev libx264-dev libfontconfig1-dev libcairo2-dev libgdk-pixbuf2.0-dev libpango1.0-dev libgtk2.0-dev libgtk-3-dev libatlas-base-dev gfortran libhdf5-dev libhdf5-serial-dev libhdf5-103 python3-pyqt5 python3-dev -y

sudo apt-get install python3-opencv

- Install i2c tools

sudo apt-get install -y i2c-tools

i2cdetect -y 1

- Thermal sensor

sudo apt-get install libatlas-base-dev python-smbus

To increase your baud rate, sudo nano /boot/config.txt and find the line with dtparam=i2c_arm=on. Add i2c_arm_baudrate=400000 to the end of it, so the result should look like:

- Run sudo reboot

- Check again if the thermal sensor is connected using i2cdetect -y 1

- Install picamera library

sudo pip3 install picamera

Pass --break-system-packages flag:

If you want to ignore the warning, you can pass the --break-system-packages flag:

Ex: pip install --break-system-packages --user <foobar>

- Run this and install the library for the thermal sensor

sudo pip3 install pithermalcam --no-deps

sudo apt install python3-matplotlib

sudo apt-get install python3-scipy

sudo pip3 install RPi.GPIO --no-deps

sudo pip3 install Adafruit-Blinka

sudo pip3 install adafruit-circuitpython-mlx90640==1.3.1

sudo pip3 install cmapy --no-deps

If an older version of numpy is already installed, remove it and install the new version.

sudo apt remove python3-numpy

sudo apt install libatlas3-base

sudo pip3 install numpy==1.16.5

- Modify the pi_therm_cam.py as below.

sudo nano /usr/local/lib/python3.9/dist-packages/pithermalcam/pi_therm_cam.py

Add this in line after the get_mean_temp() function

def get_raw_mlx_temp_values(self):
    """
    Get temp of entire field of view
    """
    frame = np.zeros((24*32,))  # setup array for storing all 768 temperatures
    while True:
        try:
            self.mlx.getFrame(frame)  # read MLX temperatures into frame var
            break
        except ValueError:
            continue  # if error, just read again

    return frame

Add “return fname” line at the end of the save_image() function.

def save_image(self):
    """Save the current frame as a snapshot to the output folder."""
    fname = self.output_folder + 'pic_' + dt.datetime.now().strftime('%Y-%m-%d_%H-%M-%S') + '.jpg'
    cv2.imwrite(fname, self._image)
    self._file_saved_notification_start = time.monotonic()
    print('Thermal Image ', fname)
    return fname

Then add this row to the config.txt file:

enable_uart=1

Check if the cameras are detected

vcgencmd get_camera

- Install witty pi

wget https://www.uugear.com/repo/WittyPi3/install.sh

sudo sh install.sh

Install WiringPi

sudo apt-get install git-core

git clone https://github.com/WiringPi/WiringPi.git

cd WiringPi

git pull origin

./build

gpio -v

- Set up wi-fi on the mobile hotspot to auto-connect:

sudo nano /etc/wpa_supplicant/wpa_supplicant.conf

Add the lines below with your mobile hotspot credentials

network={

ssid="<enter_network_name>"

psk="<enter_password_for_the_network>"

key_mgmt=WPA-PSK

}

- Add this at the end of the cam_run.py and save

os.system("sudo shutdown -h now")

- Permit all to the code folder:

sudo chmod -R 777 <directory>

- Edit crontab to schedule running the capturing script.

Open cron tab: sudo crontab -e

Add this to the crontab

0 10,13,15 * * *  python3 /home/pi/Documents/camera_run.py >> /home/pi/Documents/camera_run_cronjob.log 2>&1

Logs will be saved in /home/pi/Documents/camera_run_cronjob.log

Enable cron logging: sudo nano /etc/rsyslog.conf

Uncomment the line:

# cron.* /var/log/cron.log

Then, restart the rsyslog service: sudo /etc/init.d/rsyslog restart

Check logs in: /var/log/cron.log

- Edit Witty Pi configurations of each Raspberry Pi

Open witty pi configurations and reset data

cd wittypi/

sudo ./wittyPi.sh

Enter 10 then 1

Enter 10 then 2

Enter 6 and then 5

Enter 11 to exit configuration

Edit schedule.wpi

sudo nano schedule.wpi

Erase all and paste this (Refer to the power_and_cloud_upload_schedule_2024.txt to see the schedules for each Raspberry Pi camera setup)

BEGIN   2025-03-31 09:30:00

END     2035-07-31 23:59:59

ON      M5              # keep ON state for 5  minutes for warm up (until 09:35)

OFF     M15             # keep OFF state for 15 minutes (until 09:50)

ON      M20             # keep ON state for 20 minutes (until 10:10)

OFF     H2 M40          # keep OFF state for 2 hours 40 minutes (until 12:50)

ON      M20             # keep ON state for 20 minutes (until 13:10)

OFF     H1 M40          # keep OFF state for 1 hours 40 minutes (until 14:50)

ON      M20             # keep ON state for 20 minutes (until 15:10)

OFF     H18 M20         # keep OFF state for 18 hours 40 minutes (until 09:30 next day)

Close and save it. Then run the schedule.

sudo ./runScript.sh

Check if it’s the correct date and time for the next shutdown and startup

- Shutdown

sudo shutdown -h now